17 Feb 1994

The information in the original document file is mostly outdated

and will NOT work with Windows For Workgroups 3.11.

Dis_pkt9.dos will work with WFW 3.11...but only if you follow the

immediately following directions.  For users of WFWG 3.10 the original

document is still included.

The following instructions assume that you have Windows For Workgroups

version 3.11, an ethernet card, and that you have installed WFWG 3.11

with NDIS drivers for your ethernet card.  If this is not the case with

your system, this document will be of little help to you.

Here is a copy of my protocol.ini file (found in the windows directory).

It includes settings for both an NE2000 clone card and for Microsoft's

Remote Access Server.

All entries refering to asymac and rasmac are for the Remote Access

Server and can be ignored.

The important addition to the file is the [pktdrv] section.  The

[pktdrv] section is the ONLY part of protocol.ini that you should

have to add to your protocol.ini.

[network.setup]

version=0x3110

netcard=ms$ne2clone,1,MS$NE2CLONE,3

netcard=asymac,1,ASYMAC,2

transport=ms$nwlinknb,NWLINK

transport=ms$netbeui,NETBEUI

transport=ms$ndishlp,MS$NDISHLP

lana0=ms$ne2clone,1,ms$nwlinknb

lana1=ms$ne2clone,1,ms$netbeui

lana2=asymac,1,ms$netbeui

lana3=ms$ne2clone,1,ms$ndishlp

[NWLINK]

BINDINGS=MS$NE2CLONE

[NETBEUI]

BINDINGS=MS$NE2CLONE,ASYMAC

LANABASE=1      |____

DriverName=netbeui$ |

SESSIONS=10         |____

NCBS=12                |

|

[pktdrv]               |

DriverName=PKTDRV$     |

BINDINGS=MS$NE2CLONE ; This must match the name of your adaptor

; driver in previous Bindings= statments.

; It will be different for each different

; brand and model of network card.

INTVEC=0x60	 ; This is the interupt vector that the dispkt

; packet driver will use.  You will have to give

; this number to winpkt.  i.e. winpkt 0x60

; Most other TCP/IP software will need to know

; this nubmer as well, i.e. Winsock, NCSA's config.tel

[protman]

DriverName=PROTMAN$

PRIORITY=MS$NDISHLP

[MS$NE2CLONE]

DriverName=MS2000$

INTERRUPT=5

IOBASE=0x300

[NE2000]

Adapters=MS$NE2CLONE

[MS$NDISHLP]

DriverName=ndishlp$

BINDINGS=MS$NE2CLONE

[ASYMAC]

Ports=1

MaxFrameSize=1514

CompressSend=1

CompressRecv=1

FramesPerPort=2

XonXoff=0

CompressBCast=0

[RASMAC]

Adapters=ASYMAC

You will also have to make some changes to system.ini (also in your

windows directory).  In the [network drivers] section, you must add

dis_pkt9.dos to the transport= line as below.

You also must change the LoadRMDrivers= to Yes.  Be aware that any

time you use the network setup program to change network settings

Windows will change LoadRMDrivers= back to NO.  You will have to change

it back to Yes, or dis_pkt9.dos will NOT work.

[network drivers]

netcard=ne2000.dos

transport=*netbeui,ndishlp.sys,dis_pkt9.dos

devdir=C:\WINDOWS

LoadRMDrivers=yes

After you have made all the above changes, put dis_pkt9.dos in your

Windows directory.  Then reboot your computer.

If you plan to use Trumpet Winsock, put the line:

winpkt 0x60

in your autoexec.bat after the Net Start line, remembering use the

full path unless winpkt is in your path.  Then run windows and setup

Trumpet Winsock.

If, for some strange reason, you want to use NCSA, WATTCP, or any

other dos based TCP/IP software product that uses a packet driver

interface, you can enter the command Net Logon.  This will load all

the network drivers and make the packet driver available to dos

programs.  DO NOT try to use dos based packet driver applications while

running WFWG with Winsock active.  Only one application can use the

packet driver at a time.  The Winsock interface counts as one application.

It allows you to run many winsock applications over a single packet

driver at one time.

-----------------------------------------------------------------------

Original Document file

File	DIS_PKT.DOC						3 Nov 1991

From:	Joe R. Doupnik

Utah State University

Logan Utah 84322

jrd@cc.usu.edu

(801) 750-2982 voice

(801) 750-2992 fax

What DIS_PKT.DOS does: It provides an Ethernet or a Token Ring Packet Driver

interface to programs built to operate over Packet Drivers. It talks

to NDIS (3Com/Microsoft) instead of to a lan board directly. It shares

the board with NDIS users. We call this a "shim", sitting between the

normal applications program (NetWare shells, TCP/IP, etc) and the more

hardware specific portions (NDIS in this case).

Packet Driver flavored applications		NDIS flavored applications

||					||

-------------				||

|  DIS_PKT  |				||

-------------				||

||					||

------------------------------------------

|           main module			  |

|   NDIS    ........................      |

|	    board specific driver(s)	  |

-------------------------------------------

||

--------------------------------

| Ethernet/Token Ring board(s)	|

--------------------------------

||

=================================== Ethernet/Token Ring wire

Material added to version 1.09 (as compared to v1.07) -

o Support for using old Novell 802.3 packets on the wire while using Type

8137 packets to the application. The BYU Packet Driver Novell shell

kit requires Type 8137 packets be used by the shell. The keyword "NOVELL="

in the section [pktdrv] determines whether or not conversion occurs. If a

y is placed after the NOVELL= phrase then conversion occurs:

novell = y		(case independent)

This option provides the same feature as the "-n" option of a regular

Packet Driver from the Clarkson collection.

Conversion does not occur if the right hand side is not a y, nor if the

entire line is omitted.

Conversion occurs only if the NDIS driver uses Ethernet 802.3 packets; this

also means dis_pkt provides a Packet Driver Class 1 interface to apps.

o Support for Ethernet 802.3 with 802.2 headers and Token Ring 802.5 NDIS

drivers, as Packet Driver Class 11 and 3, respectively. The NOVELL= option

does not apply to these boards. Dan Lanciani of Harvard added this part.

First sample PROTOCOL.INI file:

[protocol manager]

drivername = PROTMAN$

[pktdrv]			<-- name of this driver entry

drivername = pktdrv$		<-- formal driver name

bindings = wd8003xmac		<-- use your board's NDIS driver here

intvec = 0x60			<-- Packet Driver Int, 60h..7fh

chainvec = 0x66		<-- daisy chained Pkt Drvr Int, unused

novell = y			<-- Optional, if present and y(es) then

convert between old Novell 802.3 pkts

on the wire and Type 8137 for the app.

Omitting this line or using any other

response turns off the conversion;

default is no conversion.

[attiso]

drivername = ATTISO$		<-- Another NDIS client

bindings = wd8003xmac		<-- bound to the same harware driver

nsess = 5

ncmds = 14

use_emm = n

Western Digital EtherCard PLUS Family Adapter	<-- Ethernet board

[wd8003xmac]					<-- its ndis driver

drivername = MACWD$

irq = 7

ramaddress = 0xCA00

iobase = 0x280

receivebufsize = 1536	<-- make this a full Ethernet pkt

Sample section of CONFIG.SYS (StarGROUP material is not required):

device=c:\lanman\protman.sys /i:c:\lanman	<-- must be first

device=c:\lanman\macwd.dos			<-- WD8003E driver

device=c:\lanman\dis_pkt.dos			<-- Pkt Driver (this program)

device=c:\lanman.dos\drivers\attload.dos /Y	<-- StarGROUP NDIS

device=c:\lanman.dos\drivers\attiso\attiso.dos	<-- StarGROUP NDIS

device=c:\qemm\loadhi.sys /r:1 e:\pctcp\ifcust.sys  <-- PC/TCP stuff

device=c:\qemm\loadhi.sys /r:4 e:\pctcp\ipcust.sys  <-- etc

A second, more elaborate example, with names easier to type. We start with

file PROTOCOL.INI. Note that semicolons start comment lines.

; This is a sample protocol.ini file listing three Ethernet boards:

;	attcsma.dos is an AT&T StarLAN 10 EN100

;	elnkii.dos  is a 3Com 3C503

;	wd8003.dos  is a Western Digital WD8003E

; Only one board will be selected but the other two are present.

[protocol manager]

drivername = PROTMAN$

; Packet Driver protocol users tie in here

[pktdrv]

drivername = pktdrv$

bindings = attcsma

;	bindings= elnkii

;	bindings = wd8003

intvec = 0x60

;	chainvec = 0x66		; chaining to another Packet Driver is unused

novell = no		; do not convert packet types this time

; AT&T StarGROUP protocol stack ties in here via name ATTISO$

[attiso]

drivername = ATTISO$

bindings = attcsma

;	bindings = elnkii

;	bindings = wd8003

nsess = 5

ncmds = 14

use_emm = n

;Western Digital EtherCard PLUS Family Adapter, WD8003E in this case

[wd8003]

drivername = MACWD$

irq = 7

ramaddress = 0xCA00

iobase = 0x280

receivebufsize = 1536

;	maxtransmits = 6

;	receivebuffers = 6

;	receivechains = 6

; 3Com Etherlink II, 3C503

[elnkii]

drivername = ELNKII$

ioaddress = 0x350

interrupt = 5

transceiver = onboard

maxtransmits = 12

xmitbufs = 1

; AT&T StarLAN 10 EN100

[attcsma]

drivername = ATTCSMA$

board_type = 2

irq = 2

ioaddr = 0x360

daram = 0xD000

; End of file protocol.ini

Fragment of config.sys for the second example. Note three .dos board drivers.

device=c:\system\ramdrive.sys 1024 512 128 /E

device=c:\lanman\protman.sys /i:c:\lanman.dos\drivers\star10en

device=c:\lanman\attcsma.dos

device=c:\lanman\elnkii.dos

device=c:\lanman\macwd.dos

device=c:\lanman\dis_pkt.dos

device=c:\lanman.dos\drivers\attload.dos /Y

device=c:\lanman.dos\drivers\attiso\attiso.dos

device=c:\qemm\loadhi.sys /r:1 e:\pctcp\ifcust.sys

device=c:\qemm\loadhi.sys /r:4 e:\pctcp\ipcust.sys

device=c:\qemm\loadhi.sys /r:4 c:\netdev.sys

shell=c:\command.com  /p /e:800

It seems to be necessary to run NETBIND.EXE to get all this to be active.

Some Lan Manger network startup routines run Netbind implicitly.

Copyright notice and disclaimer from the beginning of the source code

file dis_pkt.asm:

; DIS_PKT.ASM - Adapter provides Packet Driver v1.09 interface over NDIS.

; Version 1.07  18 May 1991  by Joe R. Doupnik, Utah State Univ.

; Version 1.08  9 Aug 1991 by Dan Lanciani, ddl@harvard.harvard.edu

; Version 1.09  3 Nov 1991 by Joe R. Doupnik, Utah State Univ.

; Copyright (C) 1988 - 1991 FTP Software, Inc.

This unmodified source file and it's executable form may be used and

redistributed freely.  The source may be modified, and the source or

executable versions built from the modified source may be used and

redistributed, provided that this notice and the copyright displayed by

the exectuable remain intact, and provided that the executable displays

an additional message indicating that it has been modified, and by whom.

FTP Software Inc. releases this software "as is", with no express or

implied warranty, including, but not limited to, the implied warranties

of merchantability and fitness for a particular purpose.

USE AT YOUR OWN RISK.

To build, using Microsoft MASM 5 or later, LINK 3.64 or later, and EXE2BIN:

masm dis_pkt;

link dis_pkt;

exe2bin dis_pkt.exe dis_pkt.dos

del dis_pkt.exe

del dis_pkt.obj

Here is an excerpt from the formal NDIS v2 specification document by 3Com

and Microsoft concerning what goes into the PROTOCOL.INI file.

PROTOCOL.INI

The PROTOCOL.INI file stores configuration and binding

information for all the protocol and MAC modules in the system.

The file uses the same general format as the LANMAN.INI file.  It

consists of a series of named sections, where the section name is

in fact the module name from a module characteristics table.

Below the bracketed module name is a set of configuration

settings for the module in name=value format.

For example:

[MYNetBIOS]

Drivername = NetBIOS$

Bindings = ETHERCARD

MaxNCBs = 16

MaxSessions = 32

MaxNames = 16

The rules for PROTOCOL.INI contents are:

o Bracketed module name.  Must be the name of a protocol or MAC

module, e.g. [MYNetBIOS].  This is the name of the module as

defined in that module's characteristics table.  The name must

be 15 characters or less (not counting the brackets).  Mixed

case may be used but the Protocol Manager will convert it to

uppercase when it reads the file into memory.

o Drivername = <device driver name>.  This parameter is required

for all device driver modules.  It defines the name of the OS/2

or DOS device driver that the module is contained in.  Note

that a single device driver name may be mentioned by several

sections of the PROTOCOL.INI file, if the driver contains

multiple logical modules.  The Drivername parameter is the

recommended method by which a module searches for its module

section in the PROTOCOL.INI file to get its configuration

parameters.  This allows the module to find all relevant module

sections based on a single name intrinisic to the module

independent of the particular bracketed module name used in the

PROTOCOL.INI file.  This keyword is also required for DOS

dynamic modules like TSRs or transient application modules.

Although there is no driver name instrinsically assigned to

such modules it is required that a unique name be assigned to

this keyword for such modules anyway.  In this way the same

search mechanism used by device drivers can be used by dynamic

DOS modules to find their relevant module sections in

PROTOCOL.INI.

o Bindings = <module name> | <module name>,<module name>, . . .

This parameter is optional for protocol modules.  It is not

valid for MAC modules.  If present, it is used by the protocol

module to determine what MAC modules it will ask to bind to.

(In other words, changing this parameter in the PROTOCOL.INI

file can reconfigure a protocol to bind to a different MAC.).

The Bindings parameter may be omitted if the protocol driver

software is preconfigured to bind to a particular MAC, or if

the system will only contain one MAC and one static protocol

module.  In the latter case (only in static mode), the Protocol

Manager by default will ask the one static protocol to bind to

the one MAC.

o Other keywords and parameters.  Any other keyword=value

statements are module specific.  Keyword names must be 15

characters or less.  They may be mixed case but are converted

to uppercase when read by the Protocol Manager.  Note that

keyword names are unique within the scope of each <module name>

section and can appear within the section in any order.

o Whitespace around the equals sign is not significant, nor is

trailing white space on the line.  Except for this leading and

trailing white space, all other characters of the value string

are taken verbatim.

o A list of 0 or more parameters can appear to the right of the

equals sign.  If there are no parameters the equals sign can be

optionally omitted.  A parameter is terminated by a space, tab,

comma, or semicolon.  No parameters are interpreted by the

Protocol Manager.

o A parameter can either be up to a 31-bit signed numeric value

or a string of any length.

o A numeric parameter can be expressed either in decimal or

hexadecimal format.  All numeric parameters must start with the

characters '0' through '9' or by a + or - followed by the '0'

to'9' character.  A hexadecimal parameter must start with '0x'

or '0X' and use valid hexadecimal digits.  A non-hexadecimal

numeric parameter is treated as decimal integer.  A parameter

not surrounded by quotes and starting with 0 to 9 or + and -

followed by 0 to 9 will be assumed to be a numeric parameter.

o A string is a parameter which either starts with a non-numeric

character or is surrounded with quotes ("....").  The string is

preserved in the memory image as it appears in PROTOCOL.INI.

o A line starting with a semicolon in column 1 is a comment and

is ignored.  Blank lines are ignored too.

o Lines may be as long as required.  Continuation lines are not

supported.  Lines end with CR LF.

o Tabs, formfeeds, and spaces are considered to be white space.

The Protocol Manager supports an optional section with optional

keywords defined below:

[PROTMAN]

Drivername = PROTMAN$

Dynamic = YES or NO

PRIORITY = prot1, prot2, ...

Bindstatus = YES or NO

The bracketed module name can be any valid name as long as it is

unique within this PROTOCOL.INI.  Drivername is required and must be

assigned PROTMAN$, identifying the section as belonging to the

Protocol Manager.  None of the entries are case-sensitive.

The DYNAMIC keyword is optional.  It defaults to NO if not present.

If set to NO, the Protocol Manager operates only in the static mode

and does not support dynamic protocol drivers.  If set to YES, the

Protocol Manager operates in the dynamic mode and supports both

static and dynamic binding.

The PRIORITY keyword is optional.  If absent, then the VECTOR uses

default demultiplexing priority if multiple protocol drivers are

bound to the same MAC (see Vector Demultiplexing in Chapter 7).  If

present, the parameters on the right-hand side are presumed to be a

list of protocol module names, highest priority first.  The VECTOR

prioritizes protocol drivers for demultiplexing (if necessary)

according to their order in the list, and packets are offered to the

first protocol driver listed first.  Protocol drivers not listed are

assigned default priority AFTER those listed.  It is not necessary

that a protocol driver ever bind for it to be listed here.

The BINDSTATUS keyword is optional.  If absent, then the BindStatus

command is not supported by the Protocol Manager.  If set to YES,

then BindStatus is supported by the Protocol Manager.  The default

disable condition is a memory optimization feature primarily for DOS

environments.

;;;;;;;;;;; End of file DIS_PKT.DOC